An Assessment of Annual Load Estimation Methods in

Small Watersheds for Cross Site Comparisons

Nic Gubbins1*, Weston M Slaughter2, Michael J Vlah3, Spencer Rhea3, William H McDowell4,5, Emily S Bernhardt3, Matthew RV Ross1

(1) Department of Ecosystem Science and Sustainability, Colorado State University, Fort

Collins, CO; (2) Department of Geology, University of Maryland, College Park, MD; (3) Department of Biology, Duke University, Durham, NC; (4) Department of Natural Resources and the Environment, University of New Hampshire Main Campus, Durham NH; (5) Institute of Environment, Florida International University, Miami, FL

*Corresponding author: contact gubbins@colostate.edu

# Abstract

Rivers and streams transport dissolved materials and sediments from their watersheds, providing researchers and land managers with insights into water quality, watershed landscape health, and management strategies. Despite the widespread use of export (load) estimates, comparisons at macroscales are complicated by differences in underlying data quality and estimation methods between sites, periods, and solutes. Using high-frequency data from the Hubbard Brook Experimental Forest, the Plynlimon Research Catchments, and the National Ecological Observatory Network (NEON) we generated time series of increasingly coarse sampling frequencies and tested the sensitivity of various load estimation methods. 
We applied each estimation method to the MacroSheds dataset (macrosheds.org), generating a publicly available dataset of 26,685 site-solute-years across 146 sites and 100 solutes. Results indicate that load estimates with high sampling frequency (daily or better) and an informative concentration-discharge relationship are well suited for macroscale science efforts (errors within ~10%). Estimates based on coarse (biweekly or coarser) sampling frequencies and incompletely described and/or complex C:Q relationships showed large enough error (>50%) to suggest they would be misleading if included in macroscale efforts. Our results indicate that scientists interested in comparing load estimates should first consider (1) sensitivity of their analysis to changes in load magnitudes, (2) the underlying data frequency used to generate estimates, (3) the C:Q relationship of their solute of interest, and (4) their confidence in the completeness of that C:Q relationship over the period of study.

# Introduction

Annual stream load is the mass of solutes or sediments that moves past a point in a stream during a water year. Quantifying the magnitude, timing and form of solute exports from watershed ecosystems gives researchers key insights into how a watershed functions as a system—such as a watershed’s capacity to retain nutrients or weather bedrock. In small watersheds with relatively watertight bedrock, researchers can assume that water only leaves the watershed through evaporation, transpiration, or stream discharge (Bormann et al., 1968). If this assumption is met, researchers interested in how nutrients move through ecosystems can measure nutrient inputs from the atmosphere and soil, then measure the outputs of these nutrients in streamflow. The difference between a small watershed’s inputs and outputs  can be used to precisely and accurately estimate critical ecosystem functions, such as nutrient retention (Bormann et al., 1968). For instance, researchers interested in nutrient dynamics rely on estimates of solute export to compare watershed functions in paired catchment studies (such as Bormann et al., 1968; Likens et al., 1970; Likens and Buso, 2006). Geochemists use load estimates to constrain in-watershed weathering rates (Gaillardet et al, 1999; Maher and Chamberlain, 2014). Land managers and government decision makers rely on—and invest in—accurate estimation of solute loads to write and apply policy, such as fishery management, water treatment, and conservation efforts (Meals et al., 2013; Dodds et al., 2009; Schilling et al., 2017).  However, this kind of chemical accounting relies on accurate estimates of inputs and outputs.

Ideally, solute loads would be calculated as the product of the solute’s concentration and streamflow, integrated continuously over time, as shown in Equation 1, where  is the load,  the concentration,  the streamflow, and  the time.

In reality, environmental data are rarely truly continuous. However, after a century of collective effort, streamflow gauging and modelling can generate reliable, near-continuous estimates of discharge for most systems of interest (Kirchner et al., 2004; Aulenbach et al., 2016). To measure instantaneous load at this frequency, however, would require solute concentration measurements to be near-continuous as well. Yet this kind of data, whether directly measured or modelled, is rare due to the labor and time involved in producing the measurements (Kirchner et al., 2004; Pellerin et al., 2014; Zimmer et al., 2019). Until recently (Halliday et al., 2012), measuring most solutes required a water sample to be taken to a laboratory. As a result, chemistry samples have traditionally been taken as discrete “grab” samples that must be manually collected and analyzed off-site (Kirchner et al., 2004; Pellerin et al., 2014). Sampling frequencies, therefore, have been on the order of weekly or monthly for many watershed-ecosystem studies for more than 50 years (Buso et al., 2000).

Estimating solute loads for studies with infrequent chemical sampling requires the use of models to estimate concentrations between measurements (Kirchner et al., 2004; Pellerin et al., 2014). This introduces uncertainty into truly continuous estimates of load (Richards and Holloway, 1987; Schilling et al., 2017). In recent decades, the technology to monitor water chemistry using high-frequency environmental sensors has become cost-effective enough—for a limited range of solutes and solute proxy measurements—to be measured at the same frequencies at which discharge is modeled (Kirchner et al., 2004; Pellerin et al., 2014). However, for researchers interested in historical solute records or solutes that cannot yet be measured using sensors, there is no single, ideal method for assessing the accuracy and potential bias of load estimates (Richards and Holloway, 1987; Appling et al., 2015; Aulenbach et al., 2016). Moreover, many different methods have been used to compute these estimates (Figure 1), ranging from simple averaging or step functions (Likens et al., 1977) to complex, data-intensive statistical models (Appling et al., 2015; Zhang and Hirsch, 2019). Conflicting estimation methods further challenge confidence in comparing load estimates across sites (Appling et al., 2015; Nava et al., 2019).

Figure 1: An illustration of concentration estimation techniques (in purple) resulting from high-frequency discharge data (in red) and intermittent, discrete chemistry samples (in blue). Interpolation assumes missing concentrations are predicted best by linearly interpolating observed concentrations. The rating method assumes a strong relationship between concentration and discharge, then uses discharge to impute missing concentrations. The composite method uses the rating method, but then “forces” the imputed concentrations through all known observations. The Beale ratio estimator (not shown) instead applies a correction factor after generating an estimate with available data.

While some methods are more appropriate than others for a given system and solute, it is impossible to know with confidence what was missed in years with infrequent sampling       (Richards and Holloway, 1987; Appling et al., 2015; Nava et al., 2019). This puts water researchers, especially those interested in macroscale, cross-site comparisons, in a quandary, leading to the core question of this paper:

How do we make accurate and analysis-ready load estimates that can be compared across diverse watersheds, sampling regimes, and data density?

The challenges outlined above are exacerbated when expanding scope from a single site and solute to many of each, where methodologies and sampling frequency can vary substantially. Thus, answering our research question becomes especially important for macrosystem scientists (researchers interested in working across many systems), as they are often unable to rely on the intimate, site-specific knowledge that can inform single-site analyses. While the fundamental problems presented here cannot necessarily be solved, many recent efforts have harmonized previously scattered watershed data. For example, the CAMELS dataset is a compilation of high-quality hydrology, geospatial, and climate data for well-sampled USGS sites (Newman et al., 2015). The work the CAMELS team has done to harmonize and document these data has significantly lightened the repetitive, pre-analysis burden that macrosystem scientists face.

Many efforts have been taken to improve the accuracy of solute load estimates and provide recommendations for approaches (Richards and Holloway, 1987; Appling et al. 2015;

Aulenbach et al., 2016; Schilling et al., 2017), but those efforts have either focused on large watersheds (Pellerin et al., 2014, Appling et al., 2015; Schilling et al., 2017) or on choosing the best method possible for a given time series (Richards and Holloway, 1987; Aulenbach et al., 2016). While both areas of research are deeply important for the field at large, they do not clearly delineate when load estimates can be used in cross-site comparisons. Additionally, robust statistical methods have been developed, such as the USGS’s Weighted Regressions on Time, Discharge, and Season (WRTDS) tool (Hirsch et al., 2010) that accurately model loads at sites with at least decadal records (Hirsch et al., 2010). However, these methods require sizable amounts of data and site-specific tuning and model generation that may be prohibitively time consuming across large, spatially diverse datasets. Recent work (Smits et al., 2025) has highlighted how differing underlying study design, data quality, and ease of access hinder synthesis science. This study seeks to make more data useable for synthesis science with three goals:

- To provide clear guidance to macroscale watershed scientists about when load estimates from small studies are interoperable.

- To provide flux estimates for a synthesis dataset of small watershed studies.

- To provide a framework for classifying potentially comparable flux estimates for cross-site analyses.

# Methods

Our study used data from Hubbard Brook Experimental Forest

(HBEF) in New Hampshire, USA (Hubbard Brook Watershed Ecosystem Record, 2023) and Plynlimon Research Catchments in Wales, UK (Neal et al., 2013), and the National Ecological Observatory network (NEON) to show the relative effect of estimation method and sampling frequency on load estimation error for two solutes. Next, we applied our tested methods to the geographically diverse MacroSheds synthesis dataset (Vlah et al., 2023), providing a dataset of load estimates to be used for future investigations.

## Load estimation

We chose four common load methods for this study: linear interpolation (LI), Beale ratio, rating, and composite. These methods are archetypal of the array of methods commonly used in small-watershed ecosystem studies (Meals et al., 2013). Previous work has shown that methods should be matched to the chemodynamics and data density of the time series of interest (Aulenbach et al., 2016).

### Linear interpolation

In load estimation by linear interpolation (Figure 1, bottom left), sampled chemistry values are interpolated to match the sampling frequency of their accompanying discharge time series. Load is then computed according to Equation 2.

Where  is the load in kg/ha/year, is the interpolated concentration in mg/L, and is  the streamflow in L/s. The summed product is divided by watershed area to give the areal load in kg/ha/year. Due to its simplicity, linear interpolation is commonly used in studies, especially where load estimation methods are not the focus (Meals et al., 2013). The method has been shown to work well for frequently sampled time series (Appling et al., 2015) and in time series where concentrations are highly autocorrelated (Aulenbach et al., 2016). For example, this approach has been used to estimate fluxes of nutrients and suspended sediments in the Santa Ynez Mountains (Coombs and Melack, 2013) and in describing solute export from catchments in the Bonanza Creek LTER study site (Petrone et al., 2006). We used the “RiverLoad” R package’s      “method6” function to apply linear interpolation (Nava et al., 2019).

### Beale ratio estimator

The Beale ratio estimator is often chosen for its ability to provide unbiased load estimates      (Meals et al., 2013; Nava et al., 2019). This method uses covariance of load and discharge to scale load estimates for a given period as represented in Equation 3.

Where  is the flow,  the instantaneous load,  is the total load for when concentration was measured, and  is the total of flow for the entire year. We used the “RiverLoad” R package’s

“beale.ratio” function to apply the Beale ratio estimator, which relies on the Beale ratio as described in Beale (1962). This approach is often used by the US Environmental Protection Agency (EPA) in estimating loads (Meals et al., 2013). Current use of the Beale ratio estimator is often improved by the EPA’s AutoBeale tool, which employs an algorithm to detect the most appropriate time windows over which to calculate Beale ratios in a given dataset (Lee et al., 2019).

### Rating

The rating method (Figure 1, bottom middle) first relates concentration to discharge in log-log space with a simple linear model. Then the resulting least-squares regression line is used to generate a full time series of concentrations using the discharge time series as an input. Values are then summed for the year to generate load, just as in linear interpolation. This method has been shown to be very effective when the solute of interest has a strong C:Q relationship

(Crawford, 1996; Quilbe et al., 2006). We used the “RiverLoad” R package”s “rating” function to generate rating estimates. This method has been used to estimate nutrient loading in the Illinois River (Vieux and Moreda, 2003) and export in urban streams in Puerto Rico (McDowell et al., 2019). In cases where the rating method is appropriate, current efforts generally favor more sophisticated regression methods, such as the WRTDS tool (Lee et al., 2019).

### Composite

The composite method (Figure 1, bottom right) follows the same steps as the rating method, but the resulting daily concentration time series is corrected back to the observed values used to generate the rating. First, a rating is fit as described in the previous section. Then, residuals between each observation and the rating-derived concentration at that time are calculated. The residuals are then applied as a correction to the derived time series at each sampled point. Each residual correction is then linearly interpolated between sampled days using the “imputeTS” R package (Moritz and Bartz-Beielstein, 2017) to generate a final time series. In essence, the rating-generated time series is “forced through” all known observations. This time series is then summed to compute annual loads. The composite method has been shown to combine the strengths of linear interpolation and the rating method (Aulenbach and Hooper, 2006; Appling et al., 2015; Aulenbach et al., 2016). Our application of the composite method was adapted from Appling et al. 2015. The composite method has become a premier choice for many loading analyses. For example, it has been used to estimate sulfate export at Niwot Ridge LTER site in Colorado (Crawford, 2019) and is often favored by the US Geological Survey (Zhang and Hirsch, 2019).

## Comparing subsampled data to high-frequency data

To estimate each method’s sensitivity to solute and sampling frequency, we performed a data coarsening experiment on four time series. Nitrate as nitrogen (NO3-N) and calcium (Ca, Figure 2) from the Watershed 3 site at HBEF (42.4 hectare watershed area) and the Upper Hafren site (122 ha watershed area) at the Plynlimon Research catchments.

Figure 2: The underlying chemistry and streamflow time series used in the data coarsening experiments. Chemistry data was collected at Hubbard Brook Experimental Forest in watershed 3 using a multiparameter sonde and UV-VIS nitrate analyzer (Hubbard Brook Watershed Ecosystem Record, 2023). The NO3-N time series was measured by the nitrate analyzer directly. The Ca time series was derived from sonde measurements of specific conductance regressed against grab samples of calcium; that regression is shown inset. The specific conductance record has no missing days. Streamflow was collected using a long-running rating, v-notch weir, and stage recorder.

Both HBEF chemistry time series were collected over the 2016 water year at a 15-minute frequency using a multiparameter sonde in conjunction with a long-running stream gauge. The Ca time series was constructed by fitting a least squares regression line between specific conductance sensor readings and Ca grab samples taken at the site (Figure 2, inset). Short gaps in the 15-minute sensor and discharge records were filled by linear interpolation using the “zoo” R package (Zeileis and Grothendieck, 2005). The NO3-N time series had a mean concentration of 0.048 mg/L with a standard deviation of 0.032 mg/L. The Ca time series had a mean concentration of 0.82 mg/L with a standard deviation of 0.21 mg/L. The Ca time series was complete for the year, with no missing days in the underlying specific conductance dataset. The NO3-N time series had 14 days with incomplete data and 4 days with no data (6/19/2016–6/22/2016).

There was a tight relationship between Ca and specific conductance at Watershed 3 during the 2016 water year. The fitted linear model had an R-squared of 0.66, a slope of 0.059, and an intercept of 0.013.

The HBEF discharge time series (figure 2) is complete, with a mean flow of 8.94 Lps, a flow standard deviation of 19.20 Lps, and a yield of 2.8 x 108 liters for the year. The steep dip between 1/2/2016 and 1/10/2016 was caused by freezing at the site. Hydrologists at HBEF use the robust ensemble method for imputing streamflow during freezes by using data from nearby, non-frozen gauges, as described in See et al. (2020).

The underlying C:Q relationships for both solutes over the year at HBEF are presented in Figure 3.

Figure 3: Concentration-discharge plot for NO3-N and Ca at HBEF watershed 3 for the 2016 water year. Observations are colored by season and the least-squares regression line used in the rating and composite methods is in black.  Note the presence of hysteresis loops and seasonality that our methods do not handle. The slope of the best fit line is 0.11 with an r-squared of 0.07 for NO3-N and -0.12 with an r-squared of 0.79 for Ca. The NO3-N time series has a mean of 0.048 mg/L and a standard deviation of 0.032 mg/L. The Ca time series has a mean of 0.82 mg/L and a standard deviation of 0.21 mg/L.

There is a very weak relationship between NO3-N and discharge at watershed 3, with a log-log least-squares regression slope of 0.11 and r-squared of 0.07. In contrast, Ca showed a strong, diluting C:Q relationship, with a log-log least-squares regression slope of -0.12 and r-squared of 0.79.

Both chemistry time series from Plynlimon were collected every seven hours over the 2008 water year using auto-samplers and a long running stream gauge (Figure 4) (Neal et al., 2013). The Ca time series had a mean of 0.52 mg/L and a standard deviation of 0.12 mg/L. The NO3-N time series had a mean of 0.14 mg/L and a standard deviation of 0.06 mg/L.

Data at Plynlimon was both measured more sparsely and had more gaps than the HBEF data.

The Ca time series had 41 incomplete days, and the nitrate time series had 52 incomplete days.

Figure 4: A chemistry time series collected at Plynlimon’s Upper Hafren site using grab samples. Note the many small gaps in both time series. Streamflow was collected using a long-running rating, a weir, and a stage recorder. The flow record is complete for the year.

The underlying streamflow record at the Upper Hafren site (figure 4) was complete, with no missing days. The flow record had a mean of 107.6 Lps, a standard deviation of 134.6 Lps, and a yield of 3.4 x 109 liters for the year.

C:Q relationships at Plynlimon generally mirrored those at Hubbard Brook. Namely, NO3-N shows a complex, seasonal C:Q relationship with a weak fit and Ca shows a more consistent, diluting relationship with a stronger fit. However, the Ca fit is similarly strong at Plynlimon (R² = 0.82) as at Hubbard Brook (R² = 0.79), while the NO3 fit remains weak at both sites. NO3-N also shows a slight diluting trend at Plynlimon (best fit slope of -0.11) compared to the slight enriching trend at Hubbard Brook (best fit slope of 0.11). Both C:Q relationships are presented in figure 5.

Figure 5: Concentration-discharge plot for NO3-N and Ca at Plynlimon’s Upper Hafren site for the 2008 water year. Observations are colored by season and the least-squares regression line used in the rating and composite methods is in black. The slope of the best fit line is -0.11 with an r-squared of 0.05 for NO3-N and -0.26 with an r-squared of 0.82 for Ca. The NO3-N time series has a mean of 0.14 mg/L and a standard deviation of 0.06 mg/L. The Ca time series has a mean of 0.52 mg/L and a standard deviation of 0.12 mg/L.

### Coarsening procedure

For Hubbard Brook, we coarsened each time series from the full, 15-minute resolution up to approximately daily in one-hour increments (15 minutes, then 1.25, 2.25, …, 22.25 hours), from daily to weekly by day (sampled every day, every other day, every third day, …, every seventh day), and then at approximately 37-day and 68-day intervals, as partially illustrated in Figure 6. A similar methodology was applied to the Plynlimon time series data, excluding coarsening intervals finer than the 7-hour sampling frequency.

Figure 6: A selection of gradually coarsened nitrate time series generated from the original, 15-minute HBEF      sensor time series (black line). Coarsened daily, weekly, and bimonthly are shown as an example. The resulting concentration-discharge relationships are shown below. This process was repeated with a random starting position 100 times to create a range of possible time series from the original data.

A random starting point from within each initial coarsening interval was chosen and then every nth sample was then taken to create a coarsened time series. We then applied each method to each coarsened time series and the daily mean discharge record to generate annual flux estimates. This process was repeated 100 times to generate an envelope of possible estimates with each method.

### Calculating and comparing to “true” load

“True” load was calculated by applying the composite method to daily means of the full, high-frequency time series (as recommended in Aulenbach et al. 2016). Because the composite method also serves as our reference, its coarsening results should be read as a measure of sensitivity to sampling frequency rather than as an independent test of accuracy. We then calculated percent error by comparing true load and estimates generated with the coarsened time series.

### Comparison with a sensor-derived record

To test whether our simplified decision framework selects a defensible method without access to high-frequency data, we compared load estimates derived from routine grab samples at Hubbard Brook Watershed 3 against a sensor-derived reference. Grab-sample estimates were taken from the MacroSheds record for the site, which holds roughly 52 calcium samples per water year, and were computed with each of the methods described above. The sensor-derived reference was calculated by converting 15-minute specific conductance to calcium using the regression described above, multiplying by discharge at each timestep, and summing over the water year. Published Hubbard Brook monthly flux values (Hubbard Brook Watershed Ecosystem Record, 2023) are included for reference. The comparison covers the five water years (2013-2017) for which both the sensor record and the grab-sample record are complete.

## Application to NEON data

To test our methods at a broader scale, we performed our data coarsening methods on conductivity and turbidity data from first order NEON sites. To limit the effect of data quality on our results, we only used water years with complete records. This yielded 6 sites, which are described in table 1.

Table 1: NEON sites used for analysis. Sites were chosen based on stream order and data availability. Note that all sites are larger than HBEF watershed 3, and all but WALK are substantially larger than the Upper Hafren catchment. Watershed areas are from the MacroSheds site metadata (Vlah et al., 2023).

| Site | Area (ha) | Data |
| --- | --- | --- |
| COMO | 356.3 | Specific conductivity |
| CUPE | 427.2 | Specific conductivity, turbidity |
| KING | 1303 | Specific conductivity, turbidity |
| MAYF | 1437 | Specific conductivity, turbidity |
| TECR | 297.4 | Specific conductivity, turbidity |
| WALK | 109 | Specific conductivity, turbidity |

The NEON sites tend to be much larger than those at Hubbard Brook or Plynlimon (except for WALK). These sites are spread across the country, with varying discharge and C:Q regimes.

## Generating estimates for 146 sites

MacroSheds is a synthesis dataset of long-term biogeochemical, hydroclimatic, and geospatial data from small watershed ecosystem studies. The full dataset is available to the public at https://portal.edirepository.org/nis/mapbrowse?scope=edi&identifier=1262&revision=1, the revision used here; the latest version is linked at macrosheds.org. The dataset includes harmonized data from 93 federally funded watershed studies from across the United States (Vlah et al., 2023).

To provide flux estimates to the broader community, we applied all four of our flux estimation methods, along with a simple average, as described in Aulenbach et al. 2016, to the MacroSheds dataset. Methods were applied individually to solutes at the site-year level. A simplified application of the Aulenbach et al. 2016 decision framework was applied to each site-year to give a recommended method. Our method fit a log-log linear model between concentration and discharge for the solute year. We then calculated the autocorrelation of concentration values, the R-squared value of the model, and the autocorrelation of the model residuals. Both autocorrelations were evaluated as absolute values at lag one. Each solute year was then assigned a recommended method using cutoffs of 0.30 for the model R-squared and 0.20 for autocorrelation, as summarized in Table 2.

Table 2: Simplified application of the Aulenbach et al. (2016) decision framework, used to assign a recommended load estimation method to each solute site-year. R-squared is from a log-log linear model of concentration against discharge; both autocorrelations are absolute values at lag one.

| C:Q fit (R-squared) | Residual autocorrelation | Concentration autocorrelation | Recommended method |
| --- | --- | --- | --- |
| >0.30 | >0.20 | — | Composite |
| >0.30 | ≤0.20 | — | Rating |
| ≤0.30 | — | >0.20 | Linear interpolation |
| ≤0.30 | — | ≤0.20 | Simple average |

Only sites with 85% or more days of discharge coverage and at least one chemistry sample per water year quarter were used to calculate loads.

# Results

## Method sensitivity to actual high-frequency data

Results from the data coarsening experiments at Hubbard Brook are represented in Figures 7 and 8. Generally, all methods struggled to accurately estimate nitrate load as data became increasingly coarse.

Figure 7: Results from applying four different load estimation methods to artificially coarsened, high-frequency sensor time series of nitrate-N taken from Hubbard Brook Watershed 3. Percent error of estimated annual load from the “true” load, calculated using the full time series, is on the y-axis. Coarsened data frequency is on the x-axis. The solid line indicates the median error for that frequency, with thin lines indicating minimum and maximum. Error within 5% of truth is shaded in green and within 20% in yellow. Note that linear interpolation and the composite method are both near-unbiased at daily sampling, while the composite method and Beale ratio estimator retain the lowest bias as sampling coarsens; the rating method is poor at every frequency. All methods showed increasing variance with sample frequencies coarser than daily.

Overall, the composite method performed best for this solute year across the full range of sampling frequencies, with accurate median estimates and a relatively low range of potential estimates. Linear interpolation performed comparably with sub-weekly data, with a median error below 1% at daily sampling, but degraded more quickly than the composite method as sampling coarsened, reaching roughly -27% at bimonthly intervals against the composite method’s -16%. When data frequency was high, both concentrations and model residuals were highly autocorrelated. As expected, under these conditions, methods that leverage autocorrelation, such as linear interpolation and composite, outperform all others. As sampling becomes more infrequent and the autocorrelation of concentrations breaks down, linear interpolation becomes less accurate than the composite method. Looking at the C:Q relationship presented in Figure 3, we see that while the fit is poor, there is still an informative, enriching pattern that the composite method leverages for more accurate interpolation than simple linear interpolation. While relying on the fit alone (the rating method) produces uniformly poor results, with median errors near -35% at every sampling frequency, including residual corrections greatly improves model performance.

Figure 8: Results from applying four different load estimation methods to artificially coarsened, high-frequency sensor time series of calcium (estimated from specific conductivity) taken from Hubbard Brook watershed 3. Percent error of estimated annual load from the “true” load, calculated using the full time series, is on the y-axis. Coarsened data frequency is on the x-axis. The solid line indicates the median error for that frequency, with thin lines indicating minimum and maximum. Error within 5% of truth is shaded in green and within 20% in yellow. Note that all four methods remain within about 8% of truth at every sampling frequency for this solute year. The Beale ratio estimator shows the least bias at the coarsest frequencies, the composite and rating methods settle near -5%, and linear interpolation drifts positive as sampling coarsens.

For this solute year, every method estimated annual load to within about 8% of truth at all sampling frequencies, reflecting the strong (~0.79) R-squared of the C:Q model fit. The rating method showed a consistent negative bias (~5%), because the simple linear C:Q model does not fully capture the diluting relationship at high flows visible in Figure 3. The composite method’s residual corrections removed that bias at daily sampling, where its median error was below 1%, but converged toward the rating method’s roughly -5% as sampling coarsened and fewer observations remained to correct against. Linear interpolation was similarly accurate at daily sampling and drifted positive with coarsening, reaching about +8% at bimonthly intervals, while the Beale ratio estimator was the least biased at coarse frequencies but showed increasing variance.

For calcium, the Beale ratio estimator produced the least biased median estimates at coarse sampling frequencies, at the cost of high estimate variance. Due to the underlying assumption of covariance of discharge variance and concentration variance (Beale, 1962), the method performed more accurately on the Ca time series than the NO3-N time series, where its median error grew to roughly -19% at bimonthly sampling.

Compared to Watershed 3 at Hubbard Brook, the Upper Hafren dataset from Plynlimon has much less variable stream chemistry. As shown in Figures 9 and 10, this lower variance yielded more precise estimates across all methods. Similar to Hubbard Brook, NO3-N load at Plynlimon was best estimated by the composite and linear interpolation methods at fine sampling frequencies, with the composite method and Beale ratio estimator holding up best as data were coarsened.

Figure 9: Results from applying four different load estimation methods to artificially coarsened, high-frequency sensor time series of nitrate-N taken from Plynlimon Research Catchments’ Upper Hafren site. Percent error of estimated annual load from the “true” load, calculated using the full time series, is on the y-axis. Coarsened data frequency is on the x-axis. The solid line indicates the median error for that frequency, with thin lines indicating minimum and maximum. Error within 5% of truth is shaded in green and within 20% in yellow. Note that the composite method performs best, with the Beale ratio estimator and linear interpolation also maintaining reasonable accuracy for this year of data.

Figure 10: Results from applying four different load estimation methods to artificially coarsened, high-frequency sensor time series of Calcium, taken from Plynlimon Research Catchments’ Upper Hafren site. Percent error of estimated annual load from the “true” load, calculated using the full time series, is on the y-axis. Coarsened data frequency is on the x-axis. The solid line indicates the median error for that frequency, with thin lines indicating minimum and maximum. Error within 5% of truth is shaded in green and within 20% in yellow. Note that the composite method performed exceptionally well here, producing reliable load estimates even with monthly data.

Comparing Figures 8 and 10 highlights the diminishing importance of having an informative C:Q relationship to generate load estimates as chemistry becomes less dynamic. When stream chemistry variation is low, even a less informative C:Q relationship can be leveraged to produce more accurate load estimates. In this case, having a complete understanding of water flux is enough to produce accurate Ca load estimates even with monthly chemistry data.

## NEON analysis

We tested conductivity at six first-order NEON sites (COMO, CUPE, KING, MAYF, TECR, WALK) and turbidity at five of these (all except COMO) across multiple water years (Table 1). Conductivity results (Figures A1–A4) broadly confirmed the patterns observed at HBEF and Plynlimon: the composite and rating methods performed well across sampling frequencies, benefiting from the generally strong C:Q relationships typical of conductivity. The Beale ratio estimator produced unbiased but high-variance estimates. Linear interpolation was effective with sub-weekly data but showed large errors at coarser frequencies, particularly at sites with variable discharge such as COMO, a snowmelt-dominated system in Colorado.

Turbidity results (Figures A5–A8) revealed a markedly different pattern. All methods struggled to maintain accuracy once data were coarsened beyond a few days, with errors frequently exceeding 50% at biweekly or coarser sampling. This poor performance was driven by high variance in turbidity concentrations and weak C:Q relationships, which left little exploitable structure for any estimation method. Even the composite method, which performed well for conductivity and the HBEF/Plynlimon solutes, could not reliably reduce errors below 20% for turbidity at most sites with coarser-than-weekly sampling.

## Application to MacroSheds dataset

Applying our estimation methods to the MacroSheds dataset generated 133,425 load estimates across 146 sites and 100 solutes. After filtering for minimum sampling frequency and adequate discharge coverage, 26,685 site-solute-years met quality thresholds for reliable flux estimation. Distributions of annual solute loads of nitrate (as nitrogen) and calcium are shown in the Appendix as Figure A9, with the corresponding distribution of recommended-method estimates in Figure A10 and the spread between methods in Figure A11. The complete load calculations from each solute and site-year of MacroSheds data are available at https://doi.org/10.6084/m9.figshare.24975504.v2.

## Sensor-derived load comparison

Estimates computed from routine grab samples agreed with the sensor-derived reference across the five overlapping water years (Figure 11). The recommended method tracked the sensor record with an R-squared of 0.79 and a slope of 1.15 ± 0.34 (p = 0.04), but averaged about 16% above the sensor-derived load. The rating method and Beale ratio estimator fell closest to the reference, within about 1%, while the composite method averaged about 21% high and linear interpolation about 3% high. The same comparison is shown as an annual time series in Figure A12.

Figure 11: A comparison between load estimation methods and sensor-derived calcium load at watershed 3, Hubbard Brook Experimental Forest. Sensor-derived load is calculated using 15-minute specific conductance data, converted to calcium concentrations via regression, and multiplied by discharge to compute instantaneous load at each 15-minute timestep, then summed annually. Estimated loads were calculated using weekly, discrete grab samples. (A) Estimated vs. sensor-derived annual load with a 1:1 reference line. (B) Difference between each method’s estimate and the sensor-derived load for each water year. Published Hubbard Brook values and the estimate recommended by our application of the Aulenbach et al. (2016) decision framework are included for reference.

# Discussion

## Insights on load estimation uncertainty

Our results confirm what Aulenbach and others (2016) observed in their assessment of load estimation methods in small watersheds. Namely, that densely sampled data is usually best estimated by leveraging highly autocorrelated rating residuals (composite method) or highly autocorrelated concentrations (linear interpolation), and that informative C:Q relationships should be used where possible. Using our simplified application of Aulenbach and others’ (2016) method selection procedure yields a defensible selection of load estimation methods. As shown in Figure 11, other single methods also provide reasonable estimates. However, it would be difficult to confidently assess their relative effectiveness without a sensor derived “truth” for comparison.

The agreement shown in Figure 11 supports the view that the Aulenbach et al. (2016) decision flowchart selects a defensible method without requiring outside data. With only five overlapping water years this comparison is best read as a case study rather than a validation, and the residual bias in the recommended estimates is a reminder that a defensible choice is not always the most accurate one.

Our coarsening experiments also reinforce previous findings (Aulenbach et al., 2016) that, when there is not a strong C:Q relationship, users can rely on linear interpolation or averaging methods provided sampling is frequent. For nitrate at Hubbard Brook the low (~0.07) R-squared of the C:Q fit would lead the Aulenbach et al. (2016) guidelines to recommend exactly that, and our results support the recommendation at daily sampling, where linear interpolation matched the composite method to within 1% of truth. At coarser frequencies the recommendation no longer held, because the composite method’s residual corrections retained more of the underlying C:Q information than interpolation alone. Non-linear changes in C:Q relationships and storm responses have been found to reduce load estimate quality (Fazekas et al., 2021), which the composite method’s residual corrections help to mitigate. Users with coarse data interested primarily in reducing bias, rather than error, should consider using Beale derived estimates. When there is a strong C:Q relationship, researchers should use a method that leverages it for greater accuracy.

Comparing the results from NO3-N and Ca across methods confirms that C:Q model quality is a useful, though imperfect, guide for method selection. For Ca, the strong C:Q fit enables the composite method to produce estimates within about 5% of truth across all sampling frequencies, consistent with the Aulenbach et al. (2016) guidelines. For NO3-N, despite the weak C:Q fit, the composite method still leverages the underlying enriching trend to improve upon both the rating method and linear interpolation at coarser frequencies. Both cases support the use of residual-corrected approaches when any informative C:Q relationship exists.

These NEON results underscore the importance of C:Q relationship strength in determining load estimation accuracy. For solutes with informative C:Q relationships (such as conductivity), methods that leverage that relationship (rating, composite) perform well even at larger, more hydrologically diverse sites. For solutes with weak or nonexistent C:Q relationships (such as turbidity), no tested method can compensate for infrequent sampling. This reinforces a key finding from the HBEF and Plynlimon experiments: method selection matters most when C:Q relationships are moderate—too weak and no method helps, too strong and most methods work.

## Challenges for macroscale science with load estimates

Our results represent an important conclusion for macroscale scientists interested in load estimation: if variance in solute chemistry is sufficiently low, knowing water flux is enough to accurately estimate solute loads. However, doing so with confidence requires high-frequency data or an explanatory, well-defined C:Q relationship.

Our efforts also highlight several important challenges for scientists working with load estimates. First, method selection is critically important for accurate load estimation. The results of our data coarsening experiment (Figures 7-10 and Figures A1-A8) clearly show that, especially at coarse sampling intervals, method selection may influence estimate accuracy by more than 30 percent, and by over 70 percent for solutes with weak C:Q relationships such as turbidity. Therefore, load estimates provided without documentation of the underlying methodology should be treated with great caution or should be recomputed from chemistry and discharge using tools like the “RiverLoad” (Nava et al., 2019) and “MacroSheds” (Vlah et al., 2023) R packages. By extension, these findings suggest that load estimate data providers should always release underlying concentration and discharge data.

Secondly, determining the direction of the C:Q relationship alone is not enough to assign confidence to underlying load estimates generated from that solute time series. Confidence in that determination, with respect to the full range of possible flow conditions, must also be assessed. A site that biases its collection towards baseflow days (which is common with nonevent supplemented sampling; Richards and Holloway, 1987) may erroneously conclude that their enriching solute’s C:Q relationship is chemostatic or has no pattern (Godsey et al., 2009; Koger et al., 2018; Moatar et al., 2017). A user making this conclusion would be tempted to use linear interpolation or the Beale ratio estimator to reduce both error and bias, which would yield load estimates that heavily underestimate true load for the year. Results from the data coarsening experiments suggest this trend is corrected at high sampling frequencies.

Put simply, truly assessing confidence in a load estimate requires an assessment of confidence in both the C:Q relationship itself and an assessment of confidence in having the entire C:Q relationship. All the challenges discussed here point to a clear need for more long-term, high-frequency records of stream chemistry, as well as the potential utility of using sensor data to help assess the accuracy of longer-term, low-frequency sampling efforts. While it may be eventually possible to estimate loads accurately from sparse records using machine learning or other emerging computing methods, currently there is no substitute for high-quality, frequent observations.

## Framework for considering load estimates

The challenges from the previous section—ensuring proper method selection, overcoming low all-method accuracy in highly variable systems, and assessing confidence in knowledge of the entire C:Q relationship at a site—are easy to identify, but difficult to solve, especially for scientists interested in cross-site comparisons. While other researchers can rely on an intimate understanding of the history and disturbances at each site to inform their confidence in estimates, macroscale scientists must rely only on the data provided from previous studies. Important site history narratives and key assumptions may be difficult to locate in publications and metadata, if they are recorded at all. Cultivating a rich and well-informed understanding of every individual site is often not feasible, and depending on publicly accessible documentation, it may be impossible. However, excluding good data from a synthesis effort is wasteful and limits the power of analyses and the scope at which synthesis can be performed. Therefore, it is necessary to develop a framework of assessing load estimate confidence using only the site records themselves.

We propose the framework presented in Figure 12 to classify site years by ease of creating high-confidence load estimates for cross-site comparisons. As C:Q relationships have been shown to change over time (Kirchner et al., 2004; Godsey et al., 2019, Fazekas et al., 2021) and this framework is built on the assumption that the user does not have intimate site knowledge, each site-year is assessed independently. Our framework also assumes users have a complete flow record for the site and that they have chosen the most appropriate estimation method for each solute as described in Aulenbach et al. 2016.

With effort, this framework could be greatly improved by grouping data and method selection across multiple solute years with known, stable chemodynamics. Doing so would leverage the power of long-term data and allow for better-informed C:Q models, generating higher confidence estimates. However, doing so requires the user to have a reliable method to assess the stability of C:Q relationships within individual solutes and to develop methods to handle transitions between regimes. Currently, the authors are not aware of any such methods or tools.

Figure 12: A conceptual flowchart for classifying confidence in load estimates, assuming a complete discharge time series and method selection as described in Aulenbach et al. 2016. C:Q coverage refers to the completeness of concentration observations relative to possible flow conditions. Sampling frequency is the interval at which chemistry is sampled at the site. Hydrologic regime is either defined as flashy (stormflow dominated) or not flashy (baseflow dominated). Site-years with total C:Q coverage and high sampling frequency make it ‘simple’ to produce quality estimates. Non-chemostatic site-years with incomplete C:Q coverage have ‘medium’ difficulty producing quality estimates or require ‘complex’ methods and considerations.

Data binned as “simple” is likely suitable for inter-site comparisons at large scales. Data binned as “medium” should only be used for limited applications. For example, they could be used in aggregated regional estimates of weathering rates. Data binned as “complex” should not be used without the user learning more about the site. It is possible that estimates rated as “complex” have lower error than expected, especially if the site they are derived from has been long-running and uses targeted sampling. It should be noted that while error ranges are presented for each category, they are a qualitative assessment guided by the results of our experiments. Further work will be required to truly constrain expected error for such a complex problem.

While it is beyond the scope of this effort to delineate exact boundaries for each branch in Figure 12, we fully expect to be able to do so robustly as more sensor data becomes available in a variety of small watershed systems. With adequate, widely distributed sensor data, we expect the factors identified in our framework could be used to sort load estimates by relative quality. C:Q coverage can be systematically assessed by creating a ratio of sampled flows to observed flows over the year. Sampling frequency can be easily evaluated from the stream chemistry record. The shape of the C:Q relationship can be determined by fitting a log-log simple linear model between solute concentrations and discharge at the site and assessing the slope, R-squared, and residuals of the resulting fit. Chemostatic and no pattern time series could be differentiated using the ratio of a concentration time series’ standard deviation to its mean. Hydrologic regime can be assessed using the Richards-Baker flashiness index (Baker et al., 2004) or using baseflow-quickflow separation methods. While the field gathers more sensor data, we encourage the larger macroscale watershed science community to continue to test and model the effects of these variables on load estimation. Developing robust, transparent, and automatable approaches to inform load estimation methods around these decision points will facilitate scalable and reliable cross-site load comparisons in macroscale science.

While many of our proposed fundamental questions remain unresolved, we hope this analysis can be used as a roadmap to focus the community on critical knowledge gaps in cross-site load comparisons. Namely, we implore future research to continue to work on developing robust breakpoints for the decision points presented in Figure 12 (and discussed above) and on assessing the stability and applicability of solute chemodynamics across time. Doing so in rigorous, automatable ways will unlock macroscale science efforts that rely on load estimates and empower synthesis scientists to delve into the rich library of datasets available to them from past studies.

# Summary and Conclusions

This study evaluated four common solute load estimation methods—linear interpolation, Beale ratio, rating, and composite—across three monitoring networks with varying data densities and solute dynamics. By coarsening high-frequency sensor data from HBEF, Plynlimon, and NEON sites, we quantified how method choice and sampling frequency jointly determine load estimation accuracy.

Our key findings are as follows. First, for solutes with informative C:Q relationships, the composite method consistently produced accurate load estimates (within ~5% of truth) at weekly or finer sampling frequencies. For solutes with weak or complex C:Q relationships, all four methods were accurate at daily sampling, but the composite method and Beale ratio estimator degraded most gracefully as data were coarsened, while linear interpolation and the rating method did not. Second, low variance in stream chemistry substantially relaxed the requirements for accurate load estimation, as demonstrated by the Plynlimon results where even monthly data produced reliable estimates for calcium. Third, turbidity—with its high variance and weak C:Q relationships—represented a worst-case scenario where no method could compensate for infrequent sampling.

We applied all four methods to the MacroSheds synthesis dataset, generating 26,685 site-solute-years of load estimates across 146 sites and 100 solutes using the simplified Aulenbach et al. (2016) decision framework for method selection. This dataset is publicly available for use in cross-site comparisons.

We propose a decision framework (Figure 12) for classifying confidence in load estimates based on C:Q relationship strength, C:Q coverage, sampling frequency, and hydrologic regime. Site-years classified as "simple" are suitable for macroscale cross-site comparisons, while those classified as "complex" require site-specific expertise before inclusion in synthesis analyses.

Looking ahead, the growing availability of high-frequency sensor data across diverse watershed systems will enable the community to quantify the decision points in our framework more precisely. We encourage continued development of robust, automatable approaches for assessing C:Q relationship stability and for matching estimation methods to data characteristics. Doing so will support scalable and reliable cross-site load comparisons in macroscale watershed science.

# Data Availability

Water chemistry and streamflow data for this paper were sourced from MacroSheds (Vlah et al,

2023, macrosheds.org), Hubbard Brook Experimental Forest (Hubbard Brook Watershed Ecosystem Record, 2023), and Plynlimon Research Catchments (Neal et al., 2013). Versioned code and the underlying dataset and workflow used for the analysis is available at https://github.com/ecogub/RSFME. Resulting load calculations are hosted publicly at https://doi.org/10.6084/m9.figshare.24975504.v2.

# References

Appling, A. P., Leon, M. C., & McDowell, W. H. (2015). Reducing bias and quantifying uncertainty in watershed flux estimates: the R package loadflex. Ecosphere, 6(12), 269. https://doi.org/10.1890/ES14-00517.1

Aulenbach, B. T., Burns, D. A., Shanley, J. B., Yanai, R. D., Bae, K., Wild, A. D., Yang, Y., & Yi, D. (2016). Approaches to stream solute load estimation for solutes with varying dynamics from five diverse small watersheds. Ecosphere, 7(6), e01298. https://doi.org/10.1002/ecs2.1298

Aulenbach, B. T., & Hooper, R. P. (2006). The composite method: An improved method for stream-water solute load estimation. Hydrological Processes, 20(14), 3029–3047. https://doi.org/10.1002/hyp.6147

Baker, D. B., Richards, R. P., Loftus, T. T., & Kramer, J. W. (2004). A new flashiness index: Characteristics and applications to Midwestern rivers and streams. Journal of the American Water Resources Association, 40(2), 503–522. https://doi.org/10.1111/j.1752-1688.2004.tb01046.x

Beale, E. (1962). Some uses of computers in operational research. Industrielle Organisation, 31, 51–52.

Bormann, A. F. H., Likens, G. E., Fisher, D. W., & Pierce, R. S. (1968). Nutrient Loss Accelerated by Clear-Cutting of a Forest Ecosystem. Science, 159(3817), 882–884.

Buso, D. C., Likens, G. E., & Eaton, J. S. (2000). Chemistry of precipitation, streamwater, and lakewater from the Hubbard Brook Ecosystem Study: A record of sampling protocols and analytical procedures (General Technical Report NE-275, 52 p.). U.S. Department of Agriculture, Forest Service, Northeastern Research Station. http://www.fs.fed.us/ne/newtown_square/publications/technical_reports/pdfs/2000/gtrne275.pdf

Coombs, J. S., & Melack, J. M. (2013). Initial impacts of a wildfire on hydrology and suspended sediment and nutrient export in California chaparral watersheds. Hydrological Processes, 27(26), 3842–3851. https://doi.org/10.1002/HYP.9508

Crawford, C. G. (1996). Estimating mean constituent loads in rivers by the rating-curve and flow-duration, rating-curve methods. School of Public and Environmental Affairs, Indiana University.

Crawford, J. T., Hinckley, E. L. S., Litaor, M. I., Brahney, J., & Neff, J. C. (2019). Evidence for accelerated weathering and sulfate export in high alpine environments. Environmental Research Letters, 14(12). https://doi.org/10.1088/1748-9326/ab5d9c

Dodds, W. K., Bouska, W. W., Eitzmann, J. L., Pilger, T. J., Pitts, K. L., Riley, A. J., Schloesser, J. T., & Thornbrugh, D. J. (2009). Eutrophication of U. S. freshwaters: Analysis of potential economic damages. Environmental Science and Technology, 43(1), 12–19. https://doi.org/10.1021/es801217q

Fazekas, H. M., Mcdowell, W. H., Shanley, J. B., & Wymore, A. S. (2021). Climate Variability Drives Watersheds Along a Transporter-Transformer Continuum. Geophysical Research Letters. https://doi.org/10.1029/2021GL094050

Gaillardet, J., Dupré, B., Louvat, P., & Allègre, C. J. (1999). Global silicate weathering and CO2 consumption rates deduced from the chemistry of large rivers. Chemical Geology, 159, 3–30. https://doi.org/10.1016/S0009-2541(99)00031-5

Godsey, S. E., Hartmann, J., & Kirchner, J. W. (2019). Catchment chemostasis revisited: Water quality responds differently to variations in weather and climate. Hydrological Processes, 33(24), 3056–3069. https://doi.org/10.1002/hyp.13554

Godsey, S. E., Kirchner, J. W., & Clow, D. W. (2009). Concentration-discharge relationships reflect chemostatic characteristics of US catchments. Hydrological Processes, 23(13), 1844–1864. https://doi.org/10.1002/hyp.7315

Halliday, S. J., Wade, A. J., Skeffington, R. A., Neal, C., Reynolds, B., Rowland, P., Neal, M., & Norris, D. (2012). An analysis of long-term trends, seasonality and short-term dynamics in water quality data from Plynlimon, Wales. Science of the Total Environment, 434, 186–200. https://doi.org/10.1016/j.scitotenv.2011.10.052

Hirsch, R. M., Moyer, D. L., & Archfield, S. A. (2010). Weighted regressions on time, discharge, and season (WRTDS), with an application to chesapeake bay river inputs. Journal of the American Water Resources Association, 46(5), 857–880. https://doi.org/10.1111/j.1752-1688.2010.00482.x

Hubbard Brook Watershed Ecosystem Record (HBWatER). (2023). Continuous precipitation and stream chemistry data, Hubbard Brook Ecosystem Study, 1963 – ongoing (ver. 9) [Data set]. Environmental Data Initiative. https://doi.org/10.6073/pasta/f2fb8b6542106c6db534fab76decdcec (Accessed 2024-02-15).

Kirchner, J. W., Feng, X., Neal, C., & Robson, A. J. (2004). The fine structure of water-quality dynamics: The (high-frequency) wave of the future. Hydrological Processes, 18(7), 1353–1359. https://doi.org/10.1002/hyp.5537

Koger, J. M., Newman, B. D., & Goering, T. J. (2018). Chemostatic behaviour of major ions and contaminants in a semiarid spring and stream system near Los Alamos, NM, USA. Hydrological Processes, 32(11), 1709–1716. https://doi.org/10.1002/hyp.11624

Lee, C. J., Hirsch, R. M., & Crawford, C. G. (2019). An evaluation of methods for computing annual water-quality loads (Scientific Investigations Report 2019–5084). U.S. Geological Survey. https://doi.org/10.3133/sir20195084

Likens, G. E., Bormann, F. H., Johnson, N. M., Fisher, D. W., & Pierce, R. S. (1970). Effects of Forest Cutting and Herbicide Treatment on Nutrient Budgets in the Hubbard Brook Watershed-Ecosystem. Ecological Monographs, 40(1), 23–47. https://doi.org/10.2307/1942440

Likens, G. E., Bormann, F. H., Pierce, R. S., Eaton, J. S., & Johnson, N. M. (1977). Biogeochemistry of a Forested Ecosystem. Springer-Verlag, New York.

Likens, G. E., & Buso, D. C. (2006). Variation in streamwater chemistry throughout the Hubbard Brook Valley. Biogeochemistry, 78(1), 1–30. https://doi.org/10.1007/s10533-005-2024-2

Maher, K., & Chamberlain, C. P. (2014). Hydrologic regulation of chemical weathering and the geologic carbon cycle. Science, 343(6178), 1502–1504. https://doi.org/10.1126/science.1250770

McDowell, W. H., McDowell, W. G., Potter, J. D., & Ramírez, A. (2019). Nutrient export and elemental stoichiometry in an urban tropical river. Ecological Applications, 29(2), 1–15. https://doi.org/10.1002/eap.1839

Meals, D. W., Richards, R. P., & Dressing, S. A. (2013). Pollutant Load Estimation for Water Quality Monitoring Projects. Tech Notes 8, 1(1), 21. https://www.epa.gov/polluted-runoff-nonpoint-source-pollution/nonpoint-source-monitoring-technical-notes

Moatar, F., Abbott, B. W., Minaudo, C., Curie, F., & Pinay, G. (2017). Elemental properties, hydrology, and biology interact to shape concentration-discharge curves for carbon, nutrients, sediment, and major ions. Water Resources Research, 53(2), 1270–1287. https://doi.org/10.1002/2016WR019635

Moritz, S., & Bartz-Beielstein, T. (2017). imputeTS: Time series missing value imputation in R. The R Journal, 9(1), 207. https://doi.org/10.32614/RJ-2017-009

Nava, V., Patelli, M., Rotiroti, M., & Leoni, B. (2019). An R package for estimating river compound load using different methods. Environmental Modelling & Software, 117, 100–108. https://doi.org/10.1016/j.envsoft.2019.03.012

Neal, C., Kirchner, J. W., & Reynolds, B. (2013). Plynlimon research catchment high-frequency hydrochemistry data. NERC Environmental Information Data Centre. https://doi.org/10.5285/551a10ae-b8ed-4ebd-ab38-033dd597a374

Newman, A. J., Clark, M. P., Sampson, K., Wood, A., Hay, L. E., Bock, A., Viger, R. J., Blodgett, D., Brekke, L., Arnold, J. R., Hopson, T., & Duan, Q. (2015). Development of a large-sample watershed-scale hydrometeorological data set for the contiguous USA: Data set characteristics and assessment of regional variability in hydrologic model performance. Hydrology and Earth System Sciences, 19(1), 209–223. https://doi.org/10.5194/hess-19-209-2015

Pellerin, B. A., Bergamaschi, B. A., Gilliom, R. J., Crawford, C. G., Saraceno, J., Frederick, C. P., Downing, B. D., & Murphy, J. C. (2014). Mississippi river nitrate loads from high frequency sensor measurements and regression-based load estimation. Environmental Science and Technology, 48(21), 12612–12619. https://doi.org/10.1021/es504029c

Petrone, K. C., Jones, J. B., Hinzman, L. D., & Boone, R. D. (2006). Seasonal export of carbon, nitrogen, and major solutes from Alaskan catchments with discontinuous permafrost. Journal of Geophysical Research: Biogeosciences, 111(2), 1–13. https://doi.org/10.1029/2005JG000055

Quilbé, R., Rousseau, A. N., Lafrance, P., Leclerc, J., & Amrani, M. (2006). Selecting a pesticide fate model at the watershed scale using a multi-criteria analysis. Water Quality Research Journal of Canada, 41(3), 283–295. https://doi.org/10.2166/wqrj.2006.032

Richards, R. P., & Holloway, J. (1987). Monte Carlo studies of sampling strategies for estimating tributary loads. Water Resources Research, 23(10), 1939–1948. https://doi.org/10.1029/WR023i010p01939

Schilling, K. E., Jones, C. S., Wolter, C. F., Liang, X., Zhang, Y. K., Seeman, A., Isenhart, T., Schnoebelen, D., & Skopec, M. (2017). Variability of nitrate-nitrogen load estimation results will make quantifying load reduction strategies difficult in Iowa. Journal of Soil and Water Conservation, 72(4), 317–325. https://doi.org/10.2489/jswc.72.4.317

See, C. R., Green, M. B., Yanai, R. D., Bailey, A. S., Campbell, J. L., & Hayward, J. (2020). Quantifying uncertainty in annual runoff due to missing data. PeerJ, 8. https://doi.org/10.7717/peerj.9531

Smits, A. P., Hall, E. K., Deemer, B. R., Scordo, F., Barbosa, C. C., Carlson, S. M., Cawley, K., et al. (2025). Too much and not enough data: Challenges and solutions for generating information in freshwater research and monitoring. Ecosphere, 16(3), e70205. https://doi.org/10.1002/ecs2.70205

Vieux, B. E., & Moreda, F. G. (2003). Nutrient loading assessment in the Illinois River using a synthetic approach. Journal of the American Water Resources Association, 39(4), 757–769. https://doi.org/10.1111/j.1752-1688.2003.tb04403.x

Vlah, M. J., Rhea, S., Bernhardt, E. S., Slaughter, W., Gubbins, N., DelVecchia, A. G., Thellman, A., & Ross, M. R. V. (2023). MacroSheds: A synthesis of long-term biogeochemical, hydroclimatic, and geospatial data from small watershed ecosystem studies. Limnology And Oceanography Letters, 8(3), 419–452. https://doi.org/10.1002/lol2.10325

Zeileis, A., & Grothendieck, G. (2005). zoo: S3 Infrastructure for Regular and Irregular Time Series. Journal of Statistical Software, 14(6). https://doi.org/10.18637/jss.v014.i06

Zhang, Q., & Hirsch, R. M. (2019). River Water-Quality Concentration and Flux Estimation Can be Improved by Accounting for Serial Correlation Through an Autoregressive Model. Water Resources Research, 55(11), 9705–9723. https://doi.org/10.1029/2019WR025338

Zimmer, M. A., Pellerin, B., Burns, D. A., & Petrochenkov, G. (2019). Temporal variability in nitrate-discharge relationships in large rivers as revealed by high-frequency data. Water Resources Research. https://doi.org/10.1029/2018WR023478

# Appendix A

Figure A1: Results from the linear interpolation method applied  to artificially coarsened, high-frequency sensor time series of conductivity taken from six NEON sites. Percent error of estimated annual load from the “true” load, calculated using the full time series, is on the y-axis. Coarsened data frequency is on the x-axis. The solid line indicates the median error for that frequency, with thin lines indicating minimum and maximum. Error within 5% of truth is shaded in green and within 20% in yellow. Note that the y-axis range varies between NEON figures to capture the full spread of error, which is greater than in the single-site HBEF and Plynlimon experiments. Note that linear interpolation is prone to high errors at the COMO and WALK sites.

Figure A2: Results from the Beale ratio method applied to artificially coarsened, high-frequency sensor time series of conductivity taken from six NEON sites. Percent error of estimated annual load from the “true” load, calculated using the full time series, is on the y-axis. Coarsened data frequency is on the x-axis. The solid line indicates the median error for that frequency, with thin lines indicating minimum and maximum. Error within 5% of truth is shaded in green and within 20% in yellow. Note that the y-axis range varies between NEON figures to capture the full spread of error, which is greater than in the single-site HBEF and Plynlimon experiments. The Beale method is effective at computing unbiased loads across a range of conditions.

Figure A3: Results from the rating method applied to artificially coarsened, high-frequency sensor time series of conductivity taken from six NEON sites. Percent error of estimated annual load from the “true” load, calculated using the full time series, is on the y-axis. Coarsened data frequency is on the x-axis. The solid line indicates the median error for that frequency, with thin lines indicating minimum and maximum. Error within 5% of truth is shaded in green and within 20% in yellow. Note that the y-axis range varies between NEON figures to capture the full spread of error, which is greater than in the single-site HBEF and Plynlimon experiments. The highly informative C:Q relationships common to conductivity yield high accuracies, even with coarse sampling frequencies.

Figure A4: Results from composite method applied to artificially coarsened, high-frequency sensor time series of conductivity taken from six NEON sites. Percent error of estimated annual load from the “true” load, calculated using the full time series, is on the y-axis. Coarsened data frequency is on the x-axis. The solid line indicates the median error for that frequency, with thin lines indicating minimum and maximum. Error within 5% of truth is shaded in green and within 20% in yellow. Note that the y-axis range varies between NEON figures to capture the full spread of error, which is greater than in the single-site HBEF and Plynlimon experiments. The composite method shows similar results to the rating method (Figure A3), with the lowest median error of any method at monthly sampling, though the two are comparable in spread at the coarsest frequencies.

Figure A5: Results from the linear interpolation method applied to artificially coarsened, high-frequency sensor time series of turbidity taken from five NEON sites. Percent error of estimated annual load from the “true” load, calculated using the full time series, is on the y-axis. Coarsened data frequency is on the x-axis. The solid line indicates the median error for that frequency, with thin lines indicating minimum and maximum. Error within 5% of truth is shaded in green and within 20% in yellow. Note that the y-axis range varies between NEON figures to capture the full spread of error, which is greater than in the single-site HBEF and Plynlimon experiments. Linear interpolation does a poor job of handling the variability in turbidity data, yielding large errors.

Figure A6: Results from the Beale ratio method applied to artificially coarsened, high-frequency sensor time series of turbidity taken from five NEON sites. Percent error of estimated annual load from the “true” load, calculated using the full time series, is on the y-axis. Coarsened data frequency is on the x-axis. The solid line indicates the median error for that frequency, with thin lines indicating minimum and maximum. Error within 5% of truth is shaded in green and within 20% in yellow. Note that the y-axis range varies between NEON figures to capture the full spread of error, which is greater than in the single-site HBEF and Plynlimon experiments. The Beale ratio estimator is the least biased of the four methods at weekly sampling, but it is not robust to the variability in turbidity: its median error reaches about -41% at monthly sampling, and over half of repetitions exceed 50% error at biweekly or coarser intervals.

Figure A7: Results from the rating method applied to artificially coarsened, high-frequency sensor time series of turbidity taken from five NEON sites. Percent error of estimated annual load from the “true” load, calculated using the full time series, is on the y-axis. Coarsened data frequency is on the x-axis. The solid line indicates the median error for that frequency, with thin lines indicating minimum and maximum. Error within 5% of truth is shaded in green and within 20% in yellow. Note that the y-axis range varies between NEON figures to capture the full spread of error, which is greater than in the single-site HBEF and Plynlimon experiments. The C:Q relationship for this solute is poor, making the uncorrected rating approach ineffective.

Figure A8: Results from the composite method applied to artificially coarsened, high-frequency sensor time series of turbidity taken from five NEON sites. Percent error of estimated annual load from the “true” load, calculated using the full time series, is on the y-axis. Coarsened data frequency is on the x-axis. The solid line indicates the median error for that frequency, with thin lines indicating minimum and maximum. Error within 5% of truth is shaded in green and within 20% in yellow. Note that the y-axis range varies between NEON figures to capture the full spread of error, which is greater than in the single-site HBEF and Plynlimon experiments. The C:Q relationship for this solute is poor, but correcting back to known observations keeps median error near zero at daily sampling and around -16% at weekly sampling, before degrading sharply at coarser intervals.

Figure A9: Histograms of annual calcium and nitrate-N load estimates in the MacroSheds dataset. The complete load calculations from each solute and site-year of MacroSheds data are available at https://doi.org/10.6084/m9.figshare.24975504.v2.

Figure A10: Distribution of annual load estimates for calcium and nitrate (as nitrogen) across the MacroSheds dataset, using the method recommended by our application of the Aulenbach et al. (2016) framework for each solute site-year.

Figure A11: Percentage spread between the highest and lowest load estimation methods for each calcium and nitrate solute site-year in the MacroSheds dataset, expressed relative to the midpoint of the two and shown on a log scale. Larger values indicate the solute years where the choice of method matters most.

Figure A12: Annual calcium load at Hubbard Brook Watershed 3 estimated by each method for water years 2013-2017, shown alongside the sensor-derived reference and the published Hubbard Brook values. This presents the comparison in Figure 11 as a time series.